PLANS, ROLES, DUTIES AND RESPONSIBILITIES OF CHIEF ENGINEER (WASTE WATER MANAGEMENT)

The Chief Engineer (WWM) is assisted by two Additional Chief Engineers, five Executive Engineers and twelve Assistant Executive Engineers along with other staff.

The main functions of this zone is maintenance of 400 mm dia and above size sewers, operating  and  maintaining the 14 existing waste water treatment plants situated in the core city.

The waste water generated in the city drains through three major principal valleys    (i) Vrishabhavathi, (ii) Koramangala & Challaghatta and (iii) Hebbal. The other minor valleys of Tavarekere, Kathriguppe, Kethamaranahalli and Arkavathi also drains in to the above major valleys. The sewers to an extent of 245 Kms are being maintained by this zone. The main objective is to intercept and divert the waste water flowing in open storm water drains and to convey the same to the respective waste water treatment plants with in the drainage zones for treatment to the required standards up to secondary levels prescribed by Karnataka State Pollution Board before letting the treated  waste water to the natural water bodies in the downstream.

PAST, PRESENT AND FUTURE PROJECTS

Past projects.

Waste water treatment plants.

This zone has constructed and maintaining the following 14 waste water treatment plants.

| 1. | Vrishabhavathi Valley | 180 MLD (Secondary) |
| --- | --- | --- |
| 2. | Vrishabhavathi Valley | 60 MLD (Tertiary) |
| 3. | Koramangala & Challaghatta Valley | 218 MLD (Secondary) |
| 4. | Koramangala & Challaghatta Valley | 30 MLD (Secondary) |
| 5. | Hebbal | 60 MLD (Secondary) |
| 6. | Yelahanka | 10 MLD (Tertiary) |
| 7. | Madiwala Tank | 4 MLD (Secondary) |
| 8. | Kempambudhi Tank | 1 MLD (Secondary) |
| 9. | Mylasandra | 75 MLD (Secondary) |
| 10. | Nagasandra | 20 MLD (Secondary) |
| 11. | Kadabeesanahalli | 50 MLD (Secondary) |
| 12. | Rajacanal | 40 MLD (Secondary) |
| 13. | K.R.Puram | 20 MLD (Secondary) |
| 14. | Jakkur | 10 MLD (Secondary) |

EAP- A Project: The Board has taken up the Environmental Action Plan –A project of replacement / rehabilitation of existing sewerage system under National River Conservation Plan scheme with 70 % financial grant from Government of India and 30% financial grant from Government of Karnataka. This scheme was complete during 2010 at a cost of     Rs.54.00 crore. After completion of this work about 40 MLD of waste water flowing in storm water drain has been diverted to the existing waste water treatment plants.

Scheme for prevention of waste water flowing in strom water drains of Hebbal valley:

The Board has taken up the scheme of zero sewage flow in Storm Water Drains of Hebbal Valley as one part of EAP-C scheme.  This project is taken up at a cost of        Rs.45.00 crore for providing and laying 18 Kms of new sewers.  This project is taken up in five packages under Mega City Revolving Funds of KUIDFC for 50% loan and 50% from the Board funds.  The works of this scheme have been completed. Now it is possible to divert about 40 MLD of sewage flowing in storm water drains to Hebbal Sewage treatment plant.

Present projects

Environmental Action Plan-B’ at an approved project cost  of  Rs.176.75 crore for a length of 70 Kms., for rehabilitation and replacement of the balance existing sewerage system of Bangalore City has been taken up under JnNURM, Government of  India.  The works included in this scheme have been taken up in 16 packages and the revised cost of the project will be  Rs.490.00 crore.  The works in respect of 12 packages have already been completed and the works in respect of 3 packages are under progress.  For balance one package due to higher tender premium, the tender was cancelled and action has been initiated to invite fresh bids. The physical progress so far achieved on this scheme is 80% and financial progress achieved is to a tune of Rs.294 crore. All the package works taken up under this scheme will be completed by the end of the year 2016. After completion of this scheme, it is possible to divert 150 MLD of waste water now flowing in Storm Water Drains to the treatment plants for the treatment.

Future projects

Environmental Action Plan –C project has been got up at an estimated cost of Rs.500 crore for rehabilitation/replacement of sewers of 74 Kms. For implementation of this project, the DPR is being prepared and it is proposed to take up this project under the financial assistance of KUIDFC and State Government. A time period of four years is required for implementation of this project when once the funding is available.

Establishing 6 new waste water treatment plants at various places;

In addition the Board has proposed to set up 6 new waste water treatment plants at a cost of Rs.1500 crore and the total capacity of these plants will be 520 MLD. The details are as follows:

- 150 MLD capacity plant at Vrishabhavathi Valley.

- 100 MLD capacity plant at Hebbal Valley.

- 150 MLD capacity plant at K&C Valley.

- 60 MLD capacity plant at Bangalore University Valley.

- 20 MLD capacity plant at K.R. Puram.

- 40 MLD capacity plant at Doddabele.

The DPRs’ for these projects are prepared and a proposal has been sent to KUIDFC and Government for funding. After receipt of funds the works will be taken up and completed in a time period of 3 years.

Additional 5 new waste water treatment plants at various places;-

Correspondence is made with L.D.A & M.O.E.F for providing suitable land area near lakes and also for funding assistance for construction of additional 5 STPs at the said locations.

- Sarakki (Jaraganahalli) 		 -  10 MLD

- Hulimavu				 -  20 MLD

- Agaram				 -  35 MLD

- Agaram (on the downstream side) 	 -  100 MLD

- Begur  					 -  10 MLD

Frequently asked questions

To rectify the (i) blockages in the sewer system, (ii) overflow of waste water from the manholes and (iii) backflow of sewage in the laterals due to blockages in the submains and trunk sewers.